BMT Exam SoSe 2019

(As always, no guarantee on completeness and correctness )

Biomedical Sensors

- Something with Schrödinger’s Cat. How does a sensor influence a system?

  - The measurement might be influenced, because the sensor is placed in the system

- How is the temperature sensed in a thermocouple?

- What is measured in a Linear Variable Differential Transformer?

Electrosurgery

- Why no electric shock in electrosurgery?

  - High frequency alternating current

- What is electrosurgery used for?

  - Coagulation, cutting of tissue

- Which statement is true with respect to neutral electrode?

  - Should not be placed on the implant

Extracorporal hemopurification systems

- Which function of the kidney does the dialyzer take over?

  - Regulating the electrolyte levels

- Which statement is wrong about requirement for dialysis membranes?

  - Maximum roughness of the membrane ( should be minimal)

- Which system is mostly used in intensive care?

  - HD

  - HF X

  - Online HD

  - …

Life Support Machine

- Cannulation for HLM

  - Withdrawal from right atrium and return in aorta

- Function of HLM

  - Oxygenate blood, pump it, decrease CO2

- Which statement is correct?

  - Centrifugal pumps are mostly used for long-term use of Heart support systems

Audiometry I & II

- Sound of 1 string is 80 dB, how loud ore 100 strings?

  - 80 dB

  - 90 dB

  - 100 dB

  - 120 dB

(very strange question because the answer was 100 dB as far as I remember, and nobody knows why…)

- What is the battery in the ear?

  - Endocholear potential together with cell resting membrane potential

- What is correct about evoked Otoacoustic emissions?

  - Test the hearing pathway up to the outer hair cells

- What is the source of the electromechanical force produced by the outer hair cell?

  - Changes in motor protein conformation caused by receptor membrane potential (oder so ähnlich?)

- Which statement is correct?

  - Something with tympanometry

- Which statement about recording of ABR is correct?

  - Need to process (average, …) it in order to find the signal

Thermofusion

- What is not an advantage of vessel sealing compared to ligature?

  - Heating of surrounding tissue

- What is correct about vessel sealing

  - Provides a fusion of larger vessel (up to 7 mm)

- Purpose of bipolar coagulation

  - Stop bleeding of capillaries and small vessels

Virtual reality

- Which compound is not a part of a virtual reality device?

  - Base station

  - Controller

  - Head-mounting device

  - Gyroscope

  - Fast graphical processing unit (GPU) X

- Which statement about Oscillopsia is correct?

  - High latency  unstationary VR-enviroment  mismatch visual vs. vestibular system (?)

- Which of the following aspects is not necessary for intraoperational navigation?

  - Position information of the surgeon X

  - fusion of MRI and CT data

  - Position information of the instruments

  - Position information of the patient

  - 3-D camera

Hygiene in biomedical technologies

- Sterilization process: 4x103 bacteria. D=90s. How many bacteria after 3 min.?

  - 4

  - 20

  - 40 X

  - 200

  - 1

- Which chemical cannot be used as Skin Disinfectant?

  - Ethanol

  - povidone-iodine

  - Formaldehyde X

  - Isopropanol

  - Chlorhexidine

- Most common transmission pathway of measles

  - Direct contact

  - Indirect contact

  - Vector

  - Aerosol X