# Exam questions BMT I WS21/22

Sonography

- What is no interaction between ultrasound waves and tissue?

  - Attenuation

  - Reflection

  - Refraction

  - Scattering

  - Amplification

Solution: e

- Higher frequency results in

  - higher resolution, lower penetration

  - lower resolution, higher penetration

  - lower resolution, lower penetration

  - higher resolution, higher penetration

  - no impact

Solution: a

- What is true for a doppler angle of 90°

- no signal

- signal dependent on blood velocity

- signal dependent on blood direction

- signal dependent on …

- …

Solution: a

Laparoscopy

4. What is the central part of the Veress needle?

- spring

- variable diameter

- larger length

- sharp needle

- …

Solution: a

5. How high should the intraabdominal pressure be?

- 10-13 mmHg

- 10-13 psi

- 10-13 bar

- 10-13 Pa

- 10-13 atm

Solution: a

6. The best position of the laparoscopic camera?

- between the trocars

- opposite of the trocars

- far away from the trocars

- always at the umbilicus

Solution: a

Respirators in anesthesia and IC medicine

7. The most appropriate lung volume per min for a person weighing 60 kg?

- less than 2 l/min

- 3.5-4.8 l/min

- nothing of all given values

- 8-9 L/min

- more than 10 L/min

Solution: b

8.  What is true about ventilation?

- Ventilation pressure needs to be limited because high pressure can lead to lung damage

- no possibility to insufflated change O2 level

- only intubation as respirators

Solution: a

9. Which statement is correct?

- artificial ventilation should not be applied in patients with lung failure due to chest trauma

- DuoPAP enables to support patients own respiration activity

Solution: b

Cardiac pacemakers and Defibrillators and IC medicine

10. Lead in the right ventricle and atrium. What can the device not have?

- Wireless Connection

- Battery

- coils that can deliver shocks

- Housing

Solution: c

11. Left ventricle pacing which vessel can be used

- Coronary sinus

- vena cava superior

- vena cava inferior

- pulmonary vein

- pulmonary artery

Solution: a

12. Order of the waves for correct sinus rhythm

- P wave - QRS complex - T wave - U wave

- QRS complex - T wave - P wave

- other combinations of ECG

Solution: a

13. Which is denoted as the primary pacemaker?

- HIS-bundle

- Purkinje fibers

- Sinus node

- AV node

- …

Solution: C

14. Which statement is wrong?

- the battery of an ICD lasts longer than for a pacemaker

15. Which statement is wrong?

- integrated light source in the center of the endoscope

- light source coupled with infrared filter in glass fiber

Solution: a

Endoscopy

16. Something about chip on a tip technology. Which statement is correct?

- Higher diameter compared to normal endoscopes

- Other possible modifications

Solution: ?

17. Which statement is correct for laparoscopy?

- valves of trocars are to prevent air to enter the peritoneum

- valves of trocars are to allow air to leave the peritoneum

- first trocar is placed in umbilicus

Solution: c

18. Something about rigid endoscopes. Which statement is wrong?

- the light source of a rigid endoscope is placed in the middle of the rigid device

- a cold light source is applicated … over fiber optic cables

- something with IR spectrum is filtered out by concave mirror

- Angle of vision can be variable

- something about the plane tip

19. and 20. about the location and size of an image for different positions of an object in front of a concave mirror

21. Which statement regarding total reflection is correct?

- refraction index of the core of an optic fiber must be higher than the one of the cladding

Solution: a

Pulmonary function testing

22. Determination of FEV1

- not dependent on the patients cooperation

23.

24. Which statement is correct?

- lung volume is dependent on sex and age

25. Something about EEG evoked responses, coherence, ….

- Each component of the evoked response…..

26. Something about pletysmography. Which statement is correct

- Must be performed in a hermetically sealed chamber

- Patients body temperature has influence on measurement

- Patients ..

- Enables to directly measure air flow through electric field

Solution: a

Data integration

What clinical standard does not exist?

- FHIR

- LOINC

- HL7

- CPR-2

- UCUM

Solution: d

Neurophysiology

27. Which is the right statement?

- MEG is based on Faradays law

- EEG is based on Biot-Savarts law

- EEG is based on Ohms law and voltage drop along axons

- EEG is based on Ohms law and voltage drop along volumes of current in the brain

- Amperes law

Solution: c

28. Which statement is correct? about infrared spectroscopic measurement of blood oxygenation and deoxygenation in the brain.

- wavelengths of 700 - 900 nm for measurements

- BOLD signals from fMRI can detect the blood deoxygenation and oxygenation.

29. wrong question, was crossed out

30. Something about innovations in MEG/EEG

- description of principle of atomic magnetometers

Solution: a

POCT

31: What is not a POCT device?

- Blood gas analysis

- ROTEM

- Multiplate platelet analyzer

- …

- blood pressure monitoring

Solution: e

32: Which of the following values for arterial blood gas analysis is correct?

- pH 6.35 - 6.45

- pCO2 75-100 mmHg

- pO2 40-50 mmHg

- BE -2 to +2

- Oxygen saturation less than 90%

Solution: d

33: Which platelet activation pathway is activated by the multiplate analyzer

- intrinsic pathway

- extrinsic pathway

- TRAP pathway

- fibrinogen

- FIIX activation

Solution: c

Clinical Laboratory Medicine

34. What is the intention behind the colored caps on the sample tubes?

- defines to which specific laboratory department it should go

- defines how fast the should be analysed

- tells the transporting personal how to carry them

- in the tubes are different anticoagulants and coagulant which are needed to proceed in the analysis later on

Solution: d

35. Test results in the clinical environment can be influenced. Thick the right statement.

- anthropologicals like age and sex have an influence on the results and should be therefore considered

36. How is the chance called, that a pathological result indeed identifies a sick person?

- diagnostic efficiency

- positive predictive value

- diagnostic sensitivity

- negative predictive value

- diagnostic specificity

Solution: b

Data integration

37. Which is not a standard used in data processing?

- LOINC

- UCUM

- SNOMED

- CPR-2

- FHIR

Solution: d

38. Types of integration

- semantic and structural

- harmonic and ?

- heterogeneous and homogeneous

Solution: a

39. What is DICOM?

- digital imaging and communications in medicine