BMT exam SS24

Stapler devices

- What trends in stapler technology?

  - Machined stapler

  - Reusability- sustainability

  - Smart staplers

  - Better patient outcomes

  - All of the above

- What is the typical shape for a stapler? -B shape

Medical robot systems

- What does robotic surgery do?

  - Usually translates surgeon movement

  - Does autonomously

  - Can do standardized procedures alone

  - Is just experimental

  - Is not used in clinics currently

- Advantage laparoscopy vs open surgery?

  - Less blood loss

- Advantage robotic surgery vs laparoscopy?

  - Shorter training curve

Audiometry

- ABR

- What happens in FAEP summation of A+B CM,SP,…?

Hygiene in Biomedicine

- What is not removed by disinfection

  - Bacterial spores

  - Virus

  - Funghi…

- What is a typical skin disinfectant?

  - Isopropanol

  - Formaldehyd

  - …

Electrosuregry

- Why no electric shock altough high voltage?

  - Use of high frequency alternating current

- What are the differences between monopolar and bipolar instruments?

  - Monopolar instruments don’t require a neutral electrode.

  - In bipolar instruments, the current flows between the two electrodes integrated in the instrument.

  - Bipolar instruments use low frequencies to prevent unwanted burns.

  - Bipolar instruments require a neutral electrode. (similar)

- What is wrong about spark formation?

  - Usually needed for cutting

  - Used to avoid carbonization of tissue

  - …

VR in medicine

- Which sentence describes VR the best?

  - Is a simulated experience that employs 3D near-eye displays and pose tracking to give the user an immersive feel of a virtual world.

  - It´s an interactive experience that combines the real world and computer-generated 3D content

  - It describes the merging of a real-world environment and a computer-generated one

  - Is a simulated experience that employs 3D near-eye displays and sounds and … the user an immersive feel of a virtual world.

  - …

- What is not an essential component of a VR headmounted device

    - Controller

    - Accelerating sensor

    - Gyroscope

    - Binocular display

    - Camera

Integrated operating room

- What is the overall goal of integrated operating rooms?

- Requirements in OR

  - Infrastructure supply power, network, section, medical gases, offer a safe connection to power and data networks, and provide a stable, yet flexible, platform for medical devices and accessories in order to maintaining efficient workflows, supporting safety, and improving hygiene

  - Infrastructure supply power, network, section, medical gases

  - …

- What people are in the sterile field in OR during surgery?

  - Surgeon, Assistant, scrub nurse, circulating nurse, Anesthetist

  - Surgeon, Assistant, scrub nurse, circulating nurse

  - Surgeon, Assistant, scrub nurse, circulating nurse, Anesthetist , OR manager

  - Surgeon, Assistant, scrub nurse

  - Surgeon, Assistant

Vessel sealing

- What is not an advantage of electrosurgery/vessel sealing?

  - 1. damage of lateral tissue

  - 2. less bleeding

  - 3. less remaining material in the body

  - 4. more easy to apply than suturing and knots tying

- Which statement about vessel sealing is correct?

  - 1. Is an umbrella term for all electronic coagulation techniques

  - 2. Is conducted with a conductive clip which remains in the body

  - 3. used for small vessels

  - 4. reliable fusion of larger vessels

  - 5. Is conducted using a monopolar electrode

- Which statement about bipolar coagulation is correct?

  - 1. used for larger vessels

  - 2. used for capillaries and smaller vessels

  - 3. Circulatory anastomosis to reduce bleeding (something like that)

  - 4. Is used to replace clips

Machine perfusion

- What is NOT a perfusion option?

  - HOPE

  - DHOPE

  - NMP

  - NRP

  - CDR

- What is wrong about HOPE?

  - Blood perfusion

  - Cheap

  - At 8-11°

  - …

- What is wrong about NMP?

  - Cheap

  - At physiological milieu

  - +/-37°C

  - Blood perfusion

HIPEC and PIPEC

- What means HIPEC

  - Hyperthermic Intraperitoneal chemotherapy

  - Hypothermic Intraperitoneal chemotherapy

  - …

  - ..

  - …

- Which sentence is correct?

  - HIPEC and PIPEC are used together

  - CRS + HIPEC is a standard procedure in…

  - CRS+ PIPEC is a standard procedure

  - PIPEC is an experimental procedure in…

- How is HIPEC used?

  - … intravenous

  - Before crs

  - Immediately or at the end of CRS

  - During CRS surgery

  - Laparoscopic

Trauma surgery

- What is true about robotic surgery?

  - Short learning curve

  - acceptance across colleagues

  - better robotic drilling

  - better patient outcome

  - …

- What is wrong about VR in context of pelvis surgery?

  - Already used a lot in clinics

  - Cheaper way than current surgical navigations techniques

  - …

  - …

  - …

- What to think off when developing a new implant/devices?

  - Needs to benefit patients

  - Easier use for surgeon

  - Cost efficient

  - Non-inferior than current gold standard

  - All of the above